### Table: Tool Purpose & Usage Conditions

| Tool | Purpose in this validation | Exact condition for use |
| --- | --- | --- |
| Wireshark | Deep packet inspection to diagnose specific issues found during testing (retransmissions, DNS delays, TCP handshake problems) | Use when a specific client, AP, or call session shows a problem (e.g. one video call freezing) and you need to see exactly what's happening at the packet level for that single flow |
| Airtool | Captures 802.11 (Wi-Fi) frames directly over the air, including management/control frames (association, authentication, beacon) — not just IP traffic | Use when validating the wireless layer itself — e.g. confirming clients associate/roam correctly between the 20 APs, or diagnosing failed connections before IP traffic even starts |
| Port Mirroring | Feeds a copy of real switch/WLC traffic to a monitoring port so it can be captured without disrupting production flow | Use on the Catalyst 9800's uplink port to non-invasively capture all 1000 clients' aggregate traffic for Wireshark/TCPDump analysis, without installing anything on client devices |
| TCPDump | Lightweight, scriptable packet capture directly on Linux-based infrastructure (e.g. a server hosting VoIP/video conferencing backend) | Use for capturing traffic at the server side of VoIP/video conferencing sessions, especially for automated/scheduled captures as part of a CI validation pipeline |
| TRex | Generates high-volume synthetic traffic at line rate to stress-test the WLC's forwarding capacity | Use to simulate the aggregate data-plane load of 1000 concurrent clients doing file downloads + web browsing, to confirm the new firmware doesn't degrade throughput under full load |
| VeriWave | Purpose-built WLAN performance testing — AP capacity, roaming, RF behavior | Use specifically to validate that each of the 20 APs can sustain its share of the 1000-client load (~50 clients/AP) and that roaming between APs works correctly on the new firmware |
| iPerf | Measures raw TCP/UDP throughput, jitter, and packet loss between two endpoints | Use to benchmark WLC-to-client throughput before and after the firmware update, as a baseline regression check |
| Multicast Hammer Tool | Generates high-volume multicast traffic to test multicast handling capacity and stability | Use if video conferencing or streaming content in this deployment relies on multicast delivery — confirms the WLC's new firmware correctly replicates multicast streams to many clients without drops |
| Octoscope | Provides a controlled, repeatable RF test chamber environment (shielded, no external interference) | Use for controlled regression testing of the firmware's RF performance (signal handling, interference resilience) in a lab setting, isolated from real-world RF noise — needed before this goes anywhere near the live 1000-client environment |
| IXIA – IXChariot | Application-layer performance testing — measures real VoIP call quality (MOS score, jitter, latency) and video conferencing performance end-to-end | Use specifically to validate VoIP call quality and video conferencing performance under the new firmware, since this is the tool that reports application-level experience, not just raw throughput |