| Scenario | Recommended Tool | Reason |
| --- | --- | --- |
| Measure TCP throughput between two hosts | iPerf3 | Purpose-built for exactly this — generates controlled TCP/UDP traffic between two endpoints and reports throughput, jitter, and loss directly (as you did in Module 2, Assignment 1) |
| Capture packets from a Linux server | tcpdump | Lightweight, command-line, no GUI needed — ideal for headless/remote servers accessed over SSH; captures can be saved as .pcap and later opened in Wireshark for deeper analysis |
| Validate enterprise Wi-Fi performance | VeriWave | Purpose-built for WLAN-specific testing — AP client capacity, roaming behavior, RF interference — which generic tools like iPerf or TRex don't test |
| Generate high-volume traffic | TRex | DPDK-based, capable of line-rate traffic generation (multi-Gbps) for stress-testing infrastructure, far beyond what iPerf alone can produce |
| Analyze mirrored switch traffic | Wireshark | Best suited for deep packet inspection of a SPAN/mirror port feed — full protocol dissection, filtering, and visual analysis of traffic captured from the switch |